The manual in the centrifuge and factory setting:

At standby condition, please press “STOP” key five second then enter into the menu, now the left LED display the function charter, right LED display the data.

After you enter into the manual, you can change the data if you press the speed key, and press the STOP key to enter next menu, the setting  is as follows:

| LcoK---000 | Parameter Lock | Factory value | 123 |
| --- | --- | --- | --- |
| tLt----010 | Indication alarm time after finished | Factory value | 003 Second |
| ucL----300 | Speed alarm (optional) | Factory value | 300 |
| ucH----001 | Speed hysteresis (Optioanl) | Factory value | 001－010 |
| Pcc----070 | Speed ratio | Factory value | 70－100 |
| Icc----060 | Speed integrated intensity | Factory value | 200－600 |
| Tcc----020 | Soft start speed changing rate | Factory value | 00.3－01.0 |
| N------001 | Speed display ratio | Factory value | 1 |
| Ptt----000 | Stop delayed pretection time | Factory value | 003－020 |
| Tse-----S | Time unit | Factory value | S |
| T-H---- 100 | Longest working time for one times | Factory value | 200 |
| FSr----999 | Highest speed setting point (20 times display value) | Factory value | 600 |
| SPL----999 | Lowest speed setting point (20 times display value) | Factory value | 50 |

After set it, you can press STOP key for 5 seconds; it will store the parameter and return to Standby Condition. If you don’t press any key, it will return to standby condition after 10 seconds.

Centrifuge working procedure:

After power on, the left LED will display “nhkj”,right LED will display “100”, it is self-test.

The left LED will display setting speed after 2 second, such as “8000”, right LED　will display working times, such as “200” seconds.

The left LED will display “STOP” after 2 seconds, right LED will display the working time, such as “200” seconds, now the centrifuge enter into IDLE condition and ready for work.

Please ensure the cover of centrifuge is closed before working.

Press start key, the right LED will display time and start to work (the rightmost digital of right LED will flash), motor will run, after the instrument detect the signal (The rightmost digital of left LED will flash), the centrifuge enter into the centrifuge working condition, the centrifuge will stop and alarm after the time is Zero.